Reason → Digitone 2

MIDI Transfer & Laptop-Free Live Performance System

Claude Code Project Brief  •  March 2026

# 1. Project Goal

Transfer humanized MIDI sequences from Reason DAW into an Elektron Digitone 2 hardware sequencer, enabling fully laptop-free live performance. Reason is used only during the transfer process — at showtime, the Digitone 2 is the brain.

## The Problem

Reason sequences 6–8 external hardware synths simultaneously via USB MIDI (TB-303, TR-8S, Korg Volca, Minilogue, Arturia Drumbrute, Tetr4, and others). Performing live with a laptop is fragile and inflexible. The Digitone 2 can replace the laptop entirely — but its sequences must be populated from Reason first, with humanized timing intact.

## The Solution

A Python script acts as a real-time MIDI bridge. It:

- Listens to Reason's MIDI output on the IAC Driver (notes on channels 1–8 + MIDI clock pulses)

- Forwards all note data transparently to the Digitone 2 via USB

- Counts MIDI clock pulses to track position within each pattern

- Sends Program Change messages at the right moment to advance the Digitone 2 to the next pattern

- Preserves all humanized timing — quantization is OFF on the Digitone during recording

# 2. System Architecture

## Signal Flow

Reason (IAC Driver Bus 1) → Python Script → Digitone 2 (USB)

The IAC Driver carries two types of data simultaneously:

- MIDI Clock pulses (0xF8, 24 per quarter note) — used by the script to track position

- Note data on channels 1–8 — forwarded transparently to the Digitone 2

## Channel Mapping

Each Reason MIDI Out Device sends on a specific channel. The Digitone 2 has each track set to receive on the matching channel. This mapping is preserved exactly through the bridge — the script does not remap channels.

| Reason MIDI Out Device | Channel | Digitone 2 Track | Hardware Synth |
| --- | --- | --- | --- |
| TB-303 | 3 | Track 3 (CHAN=3) | Roland TB-303 |
| TR-8S | 1 | Track 1 (CHAN=1) | Roland TR-8S |
| Minilogue | 2 | Track 2 (CHAN=2) | Korg Minilogue |
| Drumbrute | 4 | Track 4 (CHAN=4) | Arturia Drumbrute |
| Tetr4 | 5 | Track 5 (CHAN=5) | Dave Smith Tetr4 |
| Volca | 6 | Track 6 (CHAN=6) | Korg Volca |
| (additional) | 7–8 | Tracks 7–8 | As needed |

## Digitone 2 Pattern Architecture

A critical insight: on the Digitone 2, a Pattern is a global snapshot of all 16 tracks simultaneously — not a per-track clip. The math for a 60-minute live set:

- 128 patterns per project (Banks A–H, 16 patterns each)

- ~45 patterns for a 6-minute song at 120 BPM with 64-step patterns at 1X speed

- 2 songs fit per project comfortably using Song Mode

- ~4 project load breaks across a 60-minute set (hard stops, ~20–30 seconds each)

- TR-8S plays ambient/textural samples during project load breaks

# 3. MIDI Clock Approach (Option A)

Reason is configured to send MIDI Clock on the same IAC Driver bus as the note data. The script reads both from one port.

## Reason Setup (one-time)

- Preferences → Sync tab → MIDI Clock Output: IAC Driver Bus 1

- Tick: "Send MIDI clock while sequencer is stopped" (keeps clock flowing before play)

- Transport panel: enable "Send MIDI Clock"

## Clock Math at 120 BPM, 64-step patterns

| Event | Pulses |
| --- | --- |
| 1 quarter note | 24 pulses |
| 1 sixteenth note step | 6 pulses |
| Full 64-step pattern | 384 pulses |
| Send PC early (for Digitone queue time) | 12 pulses before end |

The script sends a Program Change message 12 pulses (half a 1/16th step) before the end of each pattern, giving the Digitone 2 time to queue the next pattern and switch seamlessly on the downbeat.

# 4. Digitone 2 Setup

## Track Channel Assignment

On each track: SYN page → set CHAN parameter to match the Reason MIDI Out Device channel for that synth. Set all 8 tracks before starting a recording session.

## MIDI Config

- SETTINGS → MIDI CONFIG → SYNC → Clock Receive: OFF

- SETTINGS → MIDI CONFIG → SYNC → Transport Receive: OFF

- SETTINGS → MIDI CONFIG → SYNC → PRG CH RECEIVE: ON

- Connect Digitone 2 via USB directly to Mac

## Recording Mode

- Hold REC + press PLAY to enter Live Record mode

- Quantization: OFF (preserves humanized timing from Reason)

- All 8 tracks record simultaneously in a single pass

## Program Change Mapping

PC messages 0–127 map directly to patterns A01–H16. No bank select needed.

| PC Value | Pattern |
| --- | --- |
| 0 | A01 |
| 1 | A02 |
| 15 | A16 |
| 16 | B01 |
| 127 | H16 |

## Song Mode vs Chains

- Song Mode: up to 99 rows per song, 16 songs per project — SAVED with project. Use this for live sets.

- Chains: up to 64 patterns — NOT saved, lost on power off. Do not use for live performance.

# 5. Recording Workflow

Recording is done song-by-song, one full pass each. All tracks record simultaneously.

## Pre-session checklist

- Reason: all MIDI Out Devices routed to IAC Driver Bus 1, each on correct channel

- Reason: MIDI Clock output set to IAC Driver Bus 1, Send MIDI Clock enabled

- Digitone 2: all 8 tracks set to correct CHAN values

- Digitone 2: MIDI config as above

- Script: configured with correct BPM, steps per pattern, port names

## Per-song recording steps

- On Digitone 2: navigate to first empty pattern (e.g. A01), enter Live Record

- Run the Python script — it will wait for clock signal

- Press Play in Reason — recording begins across all 8 tracks

- Script fires PC messages automatically at each pattern boundary

- Stop Reason when song is complete

- On Digitone 2: build Song Mode arrangement from recorded patterns

- Save project

# 6. Initial Code

Install dependencies first:

pip install mido python-rtmidi

Save as reason_to_digitone.py. Edit the CONFIG section at the top before running.

#!/usr/bin/env python3

"""

Reason → Digitone 2 MIDI Bridge

Listens to Reason's IAC Driver output (notes + MIDI clock),

forwards notes to Digitone 2, fires Program Change messages

at pattern boundaries to advance through patterns automatically.

"""

import mido

import threading

import time

import sys

# ─── CONFIG ──────────────────────────────────────────────────────

# Run: python3 -c "import mido; print(mido.get_input_names())"

# to find exact port names on your system.

IAC_INPUT_PORT   = "IAC Driver Bus 1"   # Reason MIDI out + clock

DIGITONE_OUTPUT  = "Digitone 2"          # Exact name from get_output_names()

BPM              = 120

STEPS_PER_PAT    = 64       # 64-step patterns at 1X speed

PPQN             = 24       # MIDI clock pulses per quarter note

STEPS_PER_BEAT   = 4        # 16th note steps per quarter note

PC_EARLY_PULSES  = 12       # Send PC this many pulses before pattern end

START_PATTERN    = 0        # 0 = A01, 1 = A02, 16 = B01, etc.

MAX_PATTERNS     = 128      # Stop after this many patterns

# ─── HELPERS ─────────────────────────────────────────────────────

def pattern_name(index):

bank = chr(65 + (index // 16))  # A, B, C...

num  = (index % 16) + 1

return f"{bank}{num:02d}"

def pulses_per_pattern():

# Each step = PPQN / STEPS_PER_BEAT pulses

return STEPS_PER_PAT * (PPQN // STEPS_PER_BEAT)

def list_ports():

print("\nAvailable INPUT ports:")

for p in mido.get_input_names():

print(f"  {p}")

print("\nAvailable OUTPUT ports:")

for p in mido.get_output_names():

print(f"  {p}")

print()

# ─── MAIN BRIDGE ─────────────────────────────────────────────────

class DigitoneBridge:

def __init__(self):

self.pulse_count    = 0

self.pattern_index  = START_PATTERN

self.running        = False

self.pat_pulses     = pulses_per_pattern()

self.pc_trigger     = self.pat_pulses - PC_EARLY_PULSES

self.pc_sent        = False   # only send once per pattern boundary

self.channels_seen  = set()   # for pre-flight multi-track check

def run(self):

# Validate ports before opening

if IAC_INPUT_PORT not in mido.get_input_names():

print(f"ERROR: Input port '{IAC_INPUT_PORT}' not found.")

list_ports()

sys.exit(1)

if DIGITONE_OUTPUT not in mido.get_output_names():

print(f"ERROR: Output port '{DIGITONE_OUTPUT}' not found.")

list_ports()

sys.exit(1)

# Pre-flight report

total = MAX_PATTERNS - START_PATTERN

print(f"\n=== Reason → Digitone 2 Bridge ===")

print(f"Patterns to record : {pattern_name(START_PATTERN)} → {pattern_name(START_PATTERN + total - 1)}")

print(f"Pulses per pattern : {self.pat_pulses}")

print(f"PC fires at pulse  : {self.pc_trigger} (early by {PC_EARLY_PULSES})")

print(f"Waiting for MIDI clock from Reason... (press play in Reason)")

print()

with mido.open_input(IAC_INPUT_PORT) as inport, \

mido.open_output(DIGITONE_OUTPUT) as outport:

for msg in inport:

# ── CLOCK ──────────────────────────────────────

if msg.type == 'clock':

if not self.running:

continue

self.pulse_count += 1

# Fire PC early, once per boundary

if self.pulse_count >= self.pc_trigger and not self.pc_sent:

next_pat = self.pattern_index + 1

if next_pat < MAX_PATTERNS:

pc_msg = mido.Message('program_change',

program=next_pat,

channel=0)

outport.send(pc_msg)

self.pc_sent = True

print(f"  PC → {pattern_name(next_pat)} "

f"(pulse {self.pulse_count}/{self.pat_pulses})")

# Pattern boundary: reset counter

if self.pulse_count >= self.pat_pulses:

self.pulse_count  = 0

self.pattern_index += 1

self.pc_sent      = False

print(f"Now in pattern: {pattern_name(self.pattern_index)}")

if self.pattern_index >= MAX_PATTERNS:

print("All patterns filled. Stopping.")

break

# ── TRANSPORT ──────────────────────────────────

elif msg.type == 'start':

self.running     = True

self.pulse_count = 0

self.pc_sent     = False

print(f"START received. Recording into {pattern_name(self.pattern_index)}")

elif msg.type == 'stop':

self.running = False

print(f"STOP received. Last pattern: {pattern_name(self.pattern_index)}")

print(f"Channels seen: {sorted(self.channels_seen)}")

elif msg.type == 'continue':

self.running = True

print("CONTINUE received.")

# ── NOTE DATA (pass through) ────────────────────

elif msg.type in ('note_on', 'note_off',

'control_change', 'pitchwheel',

'aftertouch', 'polytouch'):

outport.send(msg)

if hasattr(msg, 'channel'):

self.channels_seen.add(msg.channel + 1)  # 1-indexed

# ─── ENTRY POINT ─────────────────────────────────────────────────

if __name__ == '__main__':

bridge = DigitoneBridge()

try:

bridge.run()

except KeyboardInterrupt:

print("\nInterrupted by user.")

# 7. Suggested Claude Code Tasks

Hand this document to Claude Code in a new project and ask it to:

## Phase 1 — Core

- Verify the script runs and connects to both ports correctly

- Test clock pulse counting accuracy against a known BPM

- Confirm Program Change timing — does the Digitone switch cleanly on the downbeat?

- Add adjustable PC_EARLY_PULSES (currently 12) — may need tuning per setup

## Phase 2 — Robustness

- Add a dry-run mode: simulate recording without connecting to Digitone (for testing timing logic)

- Add a port-scan utility command: python3 reason_to_digitone.py --list-ports

- Add pattern length override per-song (some songs may use 32-step or 128-step patterns)

- Handle Reason loop mode gracefully (Continue message after repositioning)

## Phase 3 — Live Set Tools

- Song definition file (YAML or JSON): define song name, start pattern, length in patterns, BPM

- Batch recording mode: auto-advance to next song's start pattern after each song

- Post-recording report: which patterns were written, total duration, patterns remaining in project

# 8. Key Decisions & Context

| Decision | Choice | Reason |
| --- | --- | --- |
| Clock source | Reason IAC Driver (Option A) | One port for everything; simpler than timer-based |
| Transfer method | Real-time MIDI bridge | Preserves humanized timing; no MIDI file parsing needed |
| All tracks simultaneously | Yes, single pass | Digitone records all 16 tracks at once per pattern |
| Quantization during recording | OFF on Digitone | Preserves the feel from Reason |
| Live set structure | 2 songs per project, Song Mode | Chains are not saved; Song Mode persists with project |
| Project breaks | TR-8S covers transitions | Digitone project load = hard stop ~20-30 sec |
| Channel remapping | None | Reason channels match Digitone track channels 1:1 |

Generated from conversation with Claude  •  March 2026